Security Dispatch for ERT:  518-437-8600

| Requestor: | Cell Phone #: ( ) - | Work Date: (permits good for ONE day only) |  |
| --- | --- | --- | --- |
| Company: |  |  |  |
| Location of Work: Bldg: Floor: Column: |  |  |  |
| Type of Hot Work: Soldering Brazing Welding Cutting OTHER |  |  |  |
| NY CREATES Work Sponsor: Phone Number: ( ) - |  |  |  |
| Hot Work Operator (Name and Company): |  |  | Cell ( ) - |
| Type of Equipment: Torch Arc Welder Grinder Other |  |  |  |
| Specify required PPE: The requestor acknowledges that all Hot Work Permit precautions and requirements were reviewed with the Hot Work Operator before commencement of the work. Yes No |  |  |  |

Do Not Write Below this Line - FOR NY CREATES USE ONLY- Do Not Write Below this Line.

FIRE SAFETY PRECAUTIONS - Please mark each box as items are completed.

BEFORE THE WORK - ALL of the following precautions must be taken:

Hot work is prohibited in all CLASS 1 DIV 2 areas unless approved equipment is used and / or special provisions have been made with EHS.

Cutting, soldering, welding, and/or other Hot Work equipment has been thoroughly inspected and found to be in good condition needing no repair, and free of damage or defects.

Hot Work Operator is equipped with communication device, protective equipment and is knowledgeable regarding the emergency procedures (e.g., location of nearest pull station and emergency call numbers)

The operator leak-checked all equipment prior to use (hoses, regulators, cylinders)

Floor areas under and at least 35 feet around all Hot Work Operations has been swept clean of combustible and flammable materials.

Flammable and combustible liquids have been relocated, at least 35 feet away from the Hot Work Operation

Ensure area is cordoned off with barriers, danger tape and warning/danger signs to prevent unauthorized access during Hot Work activities.

Fire watch personnel are to be equipped with multi-purpose dry chemical, portable fire extinguishers and trained in their use.

Where applicable, mark each box for the following precautions that will also be taken BEFORE the work begins:

Fire resistant shields must cover combustible floors and must be below any elevated cutting/welding operation.

Fume collector or local exhaust ventilation in work area to remove smoke/vapor.

Additional protection must be used (e.g., fire blankets, fire curtains, and/or screens)

All floor and wall openings are covered to prevent sparks/slag from traveling to other unprotected areas.

Containers in or on which Hot Work will take place have been purged of flammable vapors.

Fire Alarm System Deactivation is complete: Call Security at 518-956-7082 and complete Fire Protection System Daily Permit

(CFM-00005-F1)

Any work inside an Albany Nanotech Complex’ Hazardous Production Material (HPM) area or building must follow special precautions as stated in the NY CREATES Hot Work Procedure EHS-00029 step 8.1.5

DURING / AFTER THE WORK - The following precautions will be taken:

Fire Watch must inspect the area for at least 30 minutes after all cutting/welding ceases.

The location where this work will take place was examined before the start of Hot Work Operations and all the appropriate precautions have been taken. Hot Work cannot commence until signed off by ERT/EHS representative.

| TGMS TEAM signature (HPM AREA WORK ONLY): | Date |
| --- | --- |
| Tenant/Contractor EHS Rep or competent person signature: Print: | Date |
| NY CREATES ERT/EHS signature: Print: | Date |
| Start Date: Time: End Date: Time: |  |

The work area and all adjacent areas to which sparks and heat might have spread (including floor levels above and below and on opposite side of walls) were inspected for 30 minutes after the cutting and / or welding operations ceased for the day and were found to be fire safe.

| Fire Watch Signature: Print: | Date |
| --- | --- |
| Hot Work Operator Signature: Print: | Date |

EHS/ERT Additional Requirements:      ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________